Activities solutions for y10-03-p13

Activity 1

- Here is a list of components. Copy the name of each component into the correct place in the diagram.

|  | RAM | General register | Hard disk |
| --- | --- | --- | --- |
| Speaker | Arithmetic logic unit (ALU) | Display | Instruction register |
| Mouse | Program counter register | Keyboard |  |

- Explain why programmable computers were considered a huge step forward in digital technology.

The first computers were mechanical. They could only do one task. If another task was needed, the computers had to be rebuilt or rewired.

- The term ‘stored program concept’ is used to describe modern computers. State why this term is accurate.

Modern computers store both data and instructions in the same memory. At one time, instructions and data were kept in separate memories.

### Activity 2

- For each word in the first column, write a description in the second column. One has been done for you.

| Item | Description and which component is responsible |
| --- | --- |
| Fetch | The CPU gets data and/or instructions from a memory address in RAM. |
| Decode | The control unit determines the meaning of the instruction. |
| Execute | The instruction is executed, sometimes by the arithmetic logic unit. |
| Cycle | It’s called a cycle because when one finishes another starts. It is triggered by the Control Unit. |

- Every processor has a unique instruction set. Define what is meant by an ‘instruction set’.

The full list of operations that a microprocessor chip can carry out.

- Give three examples of operations that would be executed in the arithmetic logic unit.

Add, subtract, arithmetic shift, logical shift, AND, OR, NOT

- State what goes onto the address bus during the fetch-decode-execute cycle.

The memory address of a location in RAM

- State two items that can go onto the data bus, during the fetch-decode-execute cycle.

Data and instructions