lesson plan for y10-03-p15

| Lesson | Specification content | Details | Resources |
| --- | --- | --- | --- |
| P15 | 3.1.1 2.1.1 2.2.4 | Learning outcomes - Explain the role of buses in the fetch-decode-execute cycle - Explain the relationship between the width of the address bus and the number of memory locations that can be addressed - Calculate the number of addressable memory locations provided by an address bus of a specified width. - Know the difference between primary and secondary storage. Lesson plan - Recap key elements from past lessons (Slide 3). - Remind students that instructions and data are stored and processed in binary in RAM and moved to the CPU as needed. Highlight the fact that each memory location has a unique address (Slide 4). - Explain that the CPU is connected to other components using buses (Slide 5). Explain that they are called buses as many bits are transported through them simultaneously. Explain that there are three types of bus that are used and explain their purposes using the examples given on the slide. Ask students to complete Activity 1. (Tutor: Buses are actually too small to see without magnification, but you can see how the buses are routed outside a microprocessor by looking at the gold pins of the microprocessor. Each of the pins represents a signal line or part of a bus that connects the microprocessor to the outside world. Looking at a mother board, you can see different kinds of busses (not necessarily data/address/control) represented by series of parallel lines of gold.) - Introduce the idea of directionality (Slide 6). Buses may be bidirectional (both ways) or unidirectional (one way). Ask students to think about scenarios that justify the directionalities described on the slide. - Ask the students if they know the difference between 32- and 64-bit computers (Slide 7). Get some student responses. Explain that this number is known as the bandwidth of the CPU. Explain that 32-bit CPUs are cheaper than 64-bit, but that they can access less RAM. The bandwidth of an address bus relates to how many bits of memory can be addressed through it. - Show the 4-bit example and explain that the number of unique addresses that can be accessed is 2n (Slide 8). This is the same as the number of unique patterns that can be represented in binary numbers. Ask students to complete Activity 2. (Tutor: Students should remember these patterns as they have already done the lessons on binary. They should also remember the formula for calculating the maximum number of patterns available in any number of bits. Spec point 3.1.1 does not explicitly require students to know how the width of the address bus impacts on to the amount of addressable memory a computer has. However, they are expected to know that each location in memory has a unique binary address (3.1.1) and realise that the number of bits in the address bus determines how many unique binary patterns can be produced (2.1.1) and therefore how many memory locations can be uniquely addressed.) - Explain that 32-bit CPUs can access around 4GiB of RAM (Slide 9). (Tutor: The content of the memory in the table on the slide, from bottom to top, is ‘A’, ‘B’, ‘C’, ‘D’.) - Explain that RAM is volatile and that long-term storage is therefore required (Slide 10). Ensure that students understand that the term ‘storage’ may refer to either primary or secondary storage. (Tutor: Students may confuse the term ‘storage’ with secondary storage, i.e. hard disks. One way to help them get the concept right is to refer to memory (RAM) as primary storage, to hard disks as secondary storage, and to other types of storage (DVDs, USB sticks, etc.) as tertiary storage. The terms ‘primary’ and ‘secondary’ are important, as they demonstrate the distinction between RAM and hard disks. - Wrap up (Slides 11 & 12). Homework - See homework document. | Lesson slides Lesson activities |