homework solutions for y10-03-p16

Question 1

- Here are a list of key terms. Put each one into the storage technology column it is related to.

| Burns | Lands | Polarities |
| --- | --- | --- |
| CD | Laser | Pools |
| DVD | Magnetises | Reflective |
| Electrical current | NAND | SD/SSD Cards |
| Flash | No moving parts | Tape |
| Flash drive | North/South | Transistors |
| Hard disk | Pits |  |

| Solid-state | Optical | Magnetic |
| --- | --- | --- |
| Electrical current Flash Flash drive NAND No moving parts Pools SD/SSD Cards Transistors | Burns CD DVD Lands Laser Pits Reflective | Hard disk Magnetises North/South Polarities Tape |

- Using the words from the table, write your own definition of how each type of storage technology works. Include an explanation of how data is written and read. Use full sentences and good punctuation.

Solid-state technology

Solid-state storage devices, such as SD/SSD cards and USB flash drives, have no moving parts. They use NAND flash, which consists of lots of transistors arranged in a grid.

To write data an electrical current is applied to the device. This forces electrons through a barrier, trapping them in pools. A full pool represents 0 and an empty pool 1.

To read data, a small voltage is applied. If the electron pool is empty, the transistor turns on and a 1 is read out. If it is full, the transistor doesn’t turn on and a 0 is read out.

Optical technology

CD and DVD drives are optical storage devices. Data is stored as a series of pits and lands arranged in tracks on the surface of the disk. A land represents 1 and a pit 0.

To write data to an optical disk, a laser physically burns the surface of the disk, changing its form to become more or less reflective. Reflective areas represent lands and less reflective areas represent pits.

To read data from a disk, a laser beam is shone onto its surface. A pit reflects light more dimly than a land. The amount of light reflected off the surface is detected by a light sensor and translated into 1s and 0s.

Magnetic technology

Hard disk drives (HDDs) and tape drives use patterns of magnetisation to store data.

To write data, a write head magnetises the surface of the media to one of two polarities: north-south or south-north to represent 1 or 0 respectively.

To read data the magnetic state of the media is detected.

Question 2

Here are a set of images. Name the media and the device needed to read/write it.

|  | Media | Device required | Technology |
| --- | --- | --- | --- |
|  | Tape | Tape drive | Magnetic |
|  | DVD | DVD drive | Optical |
|  | Hard disk | Hard disk drive | Magnetic |
|  | USB flash | USB drive | Solid-state |
|  | CD | CD drive | Optical |
|  | SD card | SD card read/writer | Solid-state |