activities solutions for y10-03-p17

### Activity 1

Research examples of the three types of storage devices and complete the table below. You can find most of the information you need on this webpage:

https://www.bbc.co.uk/bitesize/guides/z67j2nb/revision/3
You will need to read it carefully to extract the part that corresponds to the questions in the first column.
For information on magnetic tapes, you can visit this webpage:
https://compscistation.com/magnetic-tape-storage-advantages-and-disadvantages/

|  | Optical | Magnetic | Solid state |
| --- | --- | --- | --- |
| Capacity How much data can this device store? Note the average capacity or give a range. | CDs: 640 MB DVDs: 4.7 GB Blu-ray disks: 50 GB | Hard drives: 500 GB – 12 TB Tape: Up to 15 TB per tape | Solid state drives: 256 GB – 4 TB USB memory sticks: 2 GB – 2 TB |
| Portability Can the device be moved around with no damage to the data it contains? Why might damage be caused? | Very portable | Internal hard drives: fixed External drives: portable Tape: Very portable | Internal hard drives: fixed External drives: portable |
| Speed How quickly can data be read from the device? How fast can data be written to the device? Why is this the case? | CDs: very slow DVDs: slow Blu-ray disks: slow | Hard drives: slow Tape drives: very slow | Solid state drives: very fast USB memory sticks: fast |

### Activity 2

For each of the following situations, state which storage type would be the most appropriate and explain why.

- An international software development company has a local division with 3,400 employees. Currently, they are working on developing a new application that can analyse data from the Internet of Things sensors on jumbo jets. Approximately a tebibyte of data is collected per flight. The company is looking for: 1) a way to store the data while it is being analysed, and 2) a way to store it when it is no longer needed in the live system.

Name and give a justification for:

  - a way to store the data while it is being analysed

A solid-state drive (SSD) would be well suited for this application because it can read and write large volumes of data at speeds of up to 3 GiB per second, making it ideal for analysing flight data quickly.

  - a way to store it when it is no longer needed in the live system

Tapes provide low-cost, high-capacity data storage which makes them ideal for long-term data archiving and backup. Their slow write/retrieval rate isn’t an issue in this case, since the data is not needed in the live system. Capacity is the main issue. You can keep adding tapes, so capacity is almost unlimited.

- Media students have taken part in a national competition. Each has produced a one-hour high-definition video of an original screen play. The judges for the competition are in different parts of the country and each will do a pre-screening of a subset of entries. They are willing to accept any type of media. The competition sponsors have asked that all submissions be sent through the post, directly to one of the judges.

Name and give a justification for a way to send the submission by physical post to the judge.

The file size of the video is likely to be about 3-4 GiB. The capacity of a DVD is more than sufficient to hold such a video. DVDs are cheap to buy and strong enough to be sent through the post. Alternatively, students could use a high-capacity flash drive (memory stick). The advantage of using a flash drive is that there would be no need to have a dedicated peripheral device to record/view the video. However, using a memory stick for this purpose would be considerably more expensive than using a DVD. There is also a greater likelihood of a memory stick being damaged in transit.

- A group of students is working on a monthly college newsletter. Due to other activities, four of the group did not have time to complete their sections at school, so must complete them at home. Unfortunately, the college does not allow access to students’ file storage from home.

Name and give a justification for a way that the students can carry the newsletter files and finish their sections of the newsletter at home.

A flash drive (memory stick) would be the best way to take the newsletter files home. Flash drives are lightweight and portable and would have sufficient capacity to hold the files. Most laptops, even old ones, have USB ports where a USB drive can be inserted, so no other device is required.

Files can then be worked on either directly on the USB drive or uploaded to the hard disk of each student’s laptop. Files can be copied back to the USB drive when they have been updated and they need to be taken back to school.

Activity 3

Complete the table to show the expressions that can be used to convert the numbers in the left column into the units listed in the middle column.

| From | To | Expression |
| --- | --- | --- |
| 1024 bytes | MiB | = 1024 / (1024 * 1024) = 1 / 1024 MiB |
| 0.75TiB | GiB | = 0.75 * 1024 GiB |
| 3.2GiB | MiB | = 3.2 * 1024 MiB |
| 1.5MiB | GiB | = 1.5 / 1024 GiB |
| 10.8KiB | bytes | = 10.8 * 1024 bytes |
| 65 536 bytes | KiB | = 65 536 / 1024 KiB |