assessment solutions for y10-03-p18

Question 1

Define what is meant by the term 'stored program concept'. (2)

One mark each, for a maximum of (2).

- Both instructions and data are stored in memory. (1)

- The contents of memory can be changed (1) to implement other functionality.

- This approach allows modern computers to be reprogrammed. (1)

Question 2

Describe the roles of the address bus and the control unit in the fetch-decode-execute cycle. (4)

One mark each, for a maximum of (2) for answers related to the address bus and (2) for answers related to the control bus.

Address bus (maximum of 2).

- Holds the address of the memory (1) location.

- Is only one way. (1)

- Can only be set by the Control Unit. (1)

Control unit (maximum of 2).

- Coordinates all other components (1).

- Decodes the fetched instruction. (1)

Question 3

Complete the diagram to show the components of a computer. Use these terms to label each component. (8)

| RAM General registers Control unit | Output Instruction register Program counter register | Input Arithmetic logic unit |
| --- | --- | --- |

Question 4

State the role of registers within the CPU. (1)

Registers provide temporary storge for instructions, addresses or items of data needed during processing.

Question 5

Explain why secondary storage is needed. (2)

Computers need secondary storage so that programs and data can be saved (1) when not executing in the CPU. This is because primary memory (RAM) is volatile (1) (i.e. it loses its contents without power).

Question 6

Describe how data is stored on a solid-state drive. (4)

One mark each, up to a maximum of (4).

- Data is stored in binary representation (1s and 0s). (1)

- A solid state drive used flash memory. (1)

- There are no moving parts. (1)

- Flash memory comprises a number of memory cells/locations/transistors arranged in a grid/an array structure. (1)

- A memory cell has two states – charged and uncharged. (1)

- Each memory cell stores a single bit value — a ‘0' if it is charged/does not conduct current and a ‘1' if it is uncharged/conducts current. (1)

- An electrical charge is placed or removed to write data to a memory cell. (1)

- All memory cells have an initial value of ‘1'. A save operation converts the value of some of the cells to ‘0' by blocking the flow of current to them. (1)

Question 7

A file is 1.7KiB.

Write an expression to show how big the file is in GiB. (3)

One mark each, up to a maximum of (3).

- 1.7 in numerator. (1)

- 1024 in denominator. (1)

- 1024 in denominator. (1)

1.7 / (1024 * 1024) GiB

Question 8

Complete the table to show the number base for these units of measurement. (2)

| Units of measure | Number base |
| --- | --- |
| kilobyte | 10 |
| kibibyte | 2 |

One mark for each correct number in the right cell, up to a maximum of (2).

Question 9

A household has a set of solar panels on the roof of their garage. There is a hardware component that records statistics about the output of the solar system, including how long it functions each day. The system takes an output reading every 10 minutes. There is no network connection to the house. Less than 2048 bytes of data are collected each day.

State and give a justification for the most appropriate secondary storage technology to use in collecting the statistics described above. (4)

One mark for ‘the most appropriate technology would be solid state’. (1)

One mark each for the following, up to a maximum of (3).

- It is portable, so it could be removed from the hardware and taken into the house. (1)

- A low capacity (1) of a few MiB would be sufficient, as the generated numbers would not fill up the solid-state drive/card very quickly.

- As recording is only once every 10 minutes, the read/write time is irrelevant. (1)

Total marks: 30