activities solutions for y10-06-P33

Activity 1

Compare copper wire and fibre-optic cable. Use the world-wide web to help you.

https://www.bbc.co.uk/bitesize/guides/zvspfcw/revision/4

|  | Copper (UTP Cat6) cable | Fibre-optic cable |
| --- | --- | --- |
| How it works | An Unshielded Twisted Pair (UTP Cat6) consists of four pairs of thin, copper wires covered in plastic insulation. These are twisted together to help reduce electrical interference. They transmit data as pulses of electricity. | Fibre-optic cables consist of one or more very thin strands of glass, surrounded by several layers of protective material, encased in an outer covering. They transmit data as pulses of light. |
| Range | Short, up to 100 metres. | Long, up to 80 Km. |
| Speed | Up to 10 Gbps. | High, up to 100 Gbps. |
| Latency | High: because copper is susceptible to interference and latency increases the further a signal travels. | Low: because fibre-optic cables are not affected by electrical interference. |
| Usage | Used on LANs such as those found in homes and schools. Copper cables are also used for land-line telephone. | Undersea fibre-optic cables carry internet and voice messages between continents. |

Activity 2

Complete the table to give a definition of the key terms in the left column.

| Key term | Definition |
| --- | --- |
| Fibre-optic cable | Cables that use thin glass rods to transmit data as pulses of light. Used over long distances. |
| Copper cable | Cables that use thin strands of metal (copper) to transmit data as pulses of electricity. Used over short distances. |
| UTP cable | Unshielded twisted pair. The most common type of copper cable used to connect devices on a LAN. It consists of four pairs of thin, copper wires covered in plastic insulation. These are twisted together to help reduce interference. |
| Fibre to the cabinet | A way to connect to broadband via a mix of fibre-optic and copper cables. Fibre-optic cabling runs to a green cabinet and copper cabling runs from there to the premises. |
| Fibre to the premises | A high-speed way to connect to broadband via a fibre-optic cable. Fibre-optic cabling runs all the way to the premises. |
| Broadband | A high-capacity transmission medium capable of transmitting multiple signals and traffic types across long distances at high speeds. |