homework solutions for y10-06-P34

Question 1

Complete the table to recommend a wireless technology (Zigbee, NFC, Bluetooth, RFID) to implement each of the scenarios in column one. Give a description of how this might work in the last column.

| Situation | Recommendation | How can this work? |
| --- | --- | --- |
| A program for a smartphone automatically shuts down applications, puts the phone on silent, and sets an alarm for eight hours. All this happens when the phone is placed on a small pad on a bedside table. | NFC | The NFC tag (on the pad on the table) can be read by the application on the phone which can run automatically. |
| A retailer has a technology-enabled mirror in their fitting rooms. When a customer tries on a garment, the mirror can show the same garment in different colours or patterns. | RFID | Each garment can have an RFID tag in it that the program in the mirror can recognise. |
| A parent simply tells a device, using their own voice, to turn access to the internet off for their child’s room upstairs. | Zigbee | The devices in the house, voice assistant and router, are connected together and can communicate. |
| A poster displaying family member photos is hung on the refrigerator. Young family members just need to hold their phones near each photo and the relevant person’s number is dialled automatically. | NFC | There could be NFC tags placed under each photo containing telephone number information. The program on the smart phone would only need to read the tag and dial the number. |
| A student has acquired a new gamepad. It easily connects to the PC and tablet to play games. | Bluetooth | Bluetooth support usually comes standard on PCs and tablets, so attaching a Bluetooth gamepad should be no trouble. |
| A device in a smart home acts as a hub that can take input from sensors and control devices, such as heaters, fans, and lights. | Zigbee | Companies such as Amazon, Samsung, Belkin, and Honeywell offer many different devices that can be connected via the Internet of Things to create a smart home. |
| A new grocery store has no checkout tills at all. Pre-registered customers browse the shelves and put items directly into their bags. All they need to do is walk through a special hoop on the way out. | RFID | Customers might scan a loyalty/ID card on the way into the store. Walking through the hoop reads all the RFID tags for each of the products in the customer’s bags, totals them up, charges the customer, and sends them a receipt. |
| A driver can use a hands-free kit to accept and make phone calls without taking their hands off the steering wheel. | Bluetooth | Hands-free kits for smartphones are in common use now all over the world. |

Question 2

- Watch this video to see a demonstration of a Zigbee-enabled smart home. https://www.youtube.com/watch?v=L0ipGcTh9v8 (2:11)

What is the device that physically causes the door to open?

A servo

- Watch this video to see how an RFID microchip can help you and your pet.

https://www.youtube.com/watch?v=C-zNnxhbVMA (up to 4:50)

- Why does a live body, like a pet’s, not reject the implanted microchip?

The chip is enclosed in a material that a pet’s body will not react to.

- Where is the owner’s contact information maintained?

On a database with a recognised company.

- Watch this video about NFC and answer the two questions below. https://www.youtube.com/watch?v=4UKGHNDhMqo (6:35)

- Give an example of how NFC can be used to protect consumers from fake goods.

Manufacturers can embed NFC chips into their products during production. Owners can then read the chip to access a website to tell them if it is a real product or a fake.

- Give an example of how NFC can be used to encourage visitors to a museum to view additional online content.

Near each museum exhibit there could be a plaque with an NFC chip that the visitor could use with their smartphone to access more information about the piece.