lesson plan for y10-06-P34

| Lesson | Specification content | Details | Resources |
| --- | --- | --- | --- |
| P34 | 4.1.4 | Learning outcomes - Describe how devices are connected on a wireless network - Compare the performance of wired and wireless LANs and give examples of situations where one is preferable to the other - Summarise the characteristics of Wi-Fi, Bluetooth, RFiD, Zigbee and NFC and give examples of their use. Lesson plan - Recap last lesson in which students learned about wired transmission types (Slide 3). - Students may have been taught about electromagnetic waves in science lessons (Slide 4). Explain that microwaves and radio waves are used to transmit data wirelessly. Show Earth Lab’s How does Wi-Fi work? video. Optional activity: demonstrate a file being transmitted over Bluetooth if your mobile phone policy allows this. - Explain that wireless transmission has some limitations including low range, interference, low bandwidth, and security concerns (Slide 5). - Remind students of the term Local Area Network (LAN). Explain that a wireless network is often referred to as a WLAN or Wireless LAN (Slide 6). Explain that they most likely have one of these in their home. - In a WLAN, there is a Wireless Access Point (WAP) that is responsible for sending and receiving data to the other devices in the network (Slide 7). The WAP is connected to the broadband router, either in the same physical box or with a cable. The router will be connected to the internet usually using a wire. Ask students to complete Activity 1. - Explain that most LANs use a combination of wired and wireless technologies (Slide 8). - Explain that Wi-Fi is not the only wireless technology (Slide 9). Students may well have encountered Bluetooth, RFID (often used in security tags in retail) and NFC (often used for contactless payment systems). Have students consider when they might have used such technologies. Carry out a quick straw poll, how many students have experience of a system such as Alexa? Explain that this uses a technology called Zigbee. Activity 2 requires research online to learn about these different technologies. - Wrap up (Slide 10). Homework - See homework document. | Lesson slides Lesson activities Used in lesson: https://www.youtube.com/watch?v=xmabFJUKMdg Activity 1 https://www.bbc.co.uk/bitesize/guides/zh4whyc/revision/4 https://www.rfwireless-world.com/Terminology/wired-network-vs-wireless-network.html https://www.rfwireless-world.com/Terminology/Advantages-and-Disadvantages-of-wired-network.html https://www.youtube.com/watch?v=BYQHw3-zh0I Activity 2 https://www.youtube.com/watch?v=BYQHw3-zh0I Used in homework: Question 2 https://www.youtube.com/watch?v=L0ipGcTh9v8 https://www.youtube.com/watch?v=C-zNnxhbVMA https://www.youtube.com/watch?v=4UKGHNDhMqo |