assessment solutions for y10-06-P36

Question 1

# Give three reasons for connecting devices in a network. (3)

One mark for each category, up to a maximum of (3):

- communications (e.g. email, instant messaging) (1)

- share resources (e.g. printers, Internet access) (1)

- entertainment (e.g. play games, stream movies, download music) (1)

- centralised support (e.g. update software, control access) (1)

- share data (e.g. files on a file server, photos on cloud storage). (1)

Question 2

Explain why protocols are needed on a network. (2)

- Protocols, the rules that govern how devices communicate (1), are needed on a network because without them data could not be understood. (1)

Question 3

# Explain how a LAN differs from a WAN. (2)

- A LAN covers a small geographical site (1), such as a home or office building, whereas a WAN covers a larger geographical site (1), such as a city, country, or the world.

Question 4

# State the reason that IPV6 is used instead of IPV4. (1)

One mark for each, up to a maximum of (1):

- IPV6 can address more devices than IPV4 (1)

- the number of unique IPV4 addresses has run out and IPV6 addresses have replaced them (1)

- IPV6 has 2128 addresses whereas IPV4 has 232 addresses. (1)

Question 5

# Give a definition for the term ‘bandwidth’. (1)

One mark for each, up to a maximum of (1):

- the capacity of the network (measured in bps) (1)

- the number of bits that can be transferred in a fixed time/second. (1)

Question 6

# Write an expression to give the time needed to transmit 17MiB file over a 12Mbps connection. (5)

One mark for each up to a maximum of (5):

- 1024 × 1024 (1) × 8 (1)

- 1000 × 1000 (1)

- 17 ÷ 12 (1)

- (1024 × 1024 × 8) in numerator and (1000 × 1000) in denominator. (1)

Example for full marks: (17 × 1024 × 1024 × 8) ÷ (12 × 1000 × 1000). (5)

Question 7

# Describe how data is transmitted on a fibre-optic cable. (2)

One mark for each, up to a maximum of (2):

- fibre-optic cables are made of glass (1)

- light waves are passed/bounced through the cables. (1)

Question 8

# State two advantages and two disadvantages of using wireless technology to connect devices on a LAN rather than a cable. (4)

One mark for each advantage, up to a maximum of (2):

- client machines can move around easily because there are no cables (1)

- as there are no cables, the cost of installation may be smaller (1)

- adding clients only involves the use of software rather than physical cables. (1)

One mark for each disadvantage, up to a maximum of (2):

- wireless is less secure because it is easier to intercept a wireless signal than one on a cabled connection (1)

- wireless has less range than cables (1)

- wireless is easily interrupted by other signals, so is less reliable (1)

- wireless has lower bandwidth/capacity, so will appear slower. (1)

Question 9

# Define the term ‘topology’. (1)

The design/layout of a network. (1)

Question 10

# To connect more than one device to the Internet, you must have the functionality of three different devices. All three may be inside the same physical box.

State the names of the three devices. (3)

One mark for each, up to a maximum of (3):

- modem (1)

- router (1)

- switch. (1)

Question 11

# Describe an Internet backbone. (2)

One for each, up to a maximum of (2):

- fibre-optic cable (1)

- connects continents/countries (1)

- undersea cables (1)

- high bandwidth. (1)

Question 12

# The Internet is not the same as the World Wide Web.

  - Define the term ‘Internet’. (1)

The physical devices that make up all of the networks connected all over the world. (1)

  - Define the term ‘World Wide Web’. (1)

A service hosted on the machines connected to the Internet to provide web pages. (1)

Question 13

# Give the names of two devices that can sit at the centre of a star topology network. (2)

# One for each, up to a maximum of (2):

- Switch (1)

- Hub. (1)

Total marks: 30