homework for y11-01-P1

Name ____________________________________________	Date __________________

Question 1

Remember that embedded systems take inputs, process those inputs and produce outputs.

An electric toothbrush has these features:

- Vibrating brush (back and forth).

- Oscillating head (around and around).

- A power button to turn toothbrush on or off.

- An LED that indicates whether it is on or off.

- An LED that flashes when it needs charging and goes off when fully charged.

- An LED collar, around the base of the brush, that lights up when too much pressure is being applied to your teeth.

Complete the table to show the inputs, processing and outputs for each embedded system. One has been done for you.

|  | Input | Processing | Output |
| --- | --- | --- | --- |
| Charging LED | Reading from charging circuit. | If charge is below minimum, then flash charging LED. If charge is between medium and full, then turn charging LED off. | Charging LED is flashed or is turned off. |
| Vibrating brush |  |  |  |
| Oscillating head |  |  |  |
| Pressure LED |  |  |  |
| Power LED |  |  |  |

Question 2

A theme park has a water slide.

At the top of the slide is a stop-go light. When the light is green, a person may enter the slide. When the light is red, a person must not enter the slide.

At the top and bottom of the slide are sensors, A and B. Sensor A is opened when a person enters the slide and sensor B is opened when the person exits the slide.

Draw a flowchart to show an algorithm for controlling this system.