lesson plan for y11-01-P3

| Lesson | Specification content | Details | Resources |  |
| --- | --- | --- | --- | --- |
| P3 | 4.1.3 4.1.6 4.1.7 | Learning outcomes - Describe how packet switching is used to transmit data between devices on the internet - Explain the purpose of an IP address. - Describe the role of routers. Lesson plan - Begin the lesson by showing this video: https://www.youtube.com/watch?v=ewrBalT_eBM and explain that attempting to send a large quantity of data over the internet (such as a web page) would take a long time and might not even work. Explain that data is broken into smaller parts and sent separately. These parts are called packets. - Ask students to think about sending a letter. What information might need to go on the outside? If it is in many parts, how will the recipient know how to put them back together? Encourage discussion and share ideas as a class. - Explain that packets have three main components, the data, the header and a footer. The header is like the outside of the envelope when sending a letter. The header contains the address of the recipient and sender, the number of the packet in the sequence, what kind of packet it is, time to live (the number of hops a packet can take before its life ends) and a checksum. Students should complete Activity 1 to draw the contents of a packet. (Tutor: Remind students that the type of packet/protocol is something they may already be familiar with – HTTP, FTP, SMTP, etc.) - Remind students that in a previous lesson they learned about IP addresses. Explain that these are used to locate the sender and recipient of packets over the internet. - Walkthrough the process of downloading an image from the internet (Slide 7). Ask students to complete Activity 2 to practice ordering the steps in this process. - Explain that routers are the traffic wardens of the internet. Their purpose is to forward packets onto another part of the internet. They usually will send each packet down the least congested route. This means that packets may take different routes to their destination. This is why it is called packet switching. Ask students to complete Activity 3. - It’s important that students bring their prior learning into this lesson, so ask if they remember some previous information. Animate Slide 9 to show the answers. Ask them to complete Activity 4 to show the hierarchy of the internet and the role of the router in it. - Wrap up. Homework - See homework document. |  | Lesson slides Lesson activities Used in lesson: https://www.youtube.com/watch?v=ewrBalT_eBM https://computer.howstuffworks.com/question525.htm https://www.youtube.com/watch?v=4z0FDwp8mK8 Used in homework: https://www.youtube.com/watch?v=PBWhzz_Gn10 https://www.pcmag.com/encyclopedia/term/hop-count https://networkencyclopedia.com/time-to-live-ttl/ |