lesson plan for y11-03-p15

| Lesson | Specification content | Details | Resources |
| --- | --- | --- | --- |
| P15 | 2.2.3 2.2.4 2.3.1 | Learning outcomes - Differentiate between analogue and digital data - Define what is meant by the terms ‘amplitude’, ‘sample rate’, ‘bit depth’, and ‘sample interval’ - Describe the process of converting analogue sound into binary data - Identify factors that affect the accuracy of the digital representation. Lesson plan (Tutor: This set of slides is quite long, but they are animated and are used instead of videos or web pages. You will be presenting the majority of information in this lesson.) - Introduce the topic by asking students to consider if their experience of a live concert would be the same as listening to a recording of the concert (Slide 3). Play the video: https://www.youtube.com/watch?v=AdQvgyJ1M10 (3:10) - Start with the science lesson about sound. Students may already know how vibrations are received in our ears and interpreted in our brains (Slide 4). Ensure they understand the definition of ‘analogue’. (Tutor: Other analogue signals include temperature, light, pressure). - Discuss the need to store analogue signals as digital data (Slide 5). Emphasise that a conversion must happen. Use the term ‘analogue to digital’ and vice versa. - Describe the process of conversion as taking place in two steps ‘sampling’ and ‘quantisation’ (Slide 6). (Tutor: There is quite a lot of subject-specific vocabulary associated with this topic. Students will work on Activity 1 throughout the lesson completing a bit at a time as you introduce each new term.) Ask students to complete rows 1 to 6 of Activity 1. - Slide 7 shows an analogue wave and introduces some of the terminology students need to be familiar with. (Tutor: Bit depth has the same quantisation issues as colour depth. It can only represent so many finite patterns, based on the number of bits allocated to it.) Ask students to complete rows 7 to 11 of Activity 1. - Slide 8 (animated) steps through the process of reading a diagram of the analogue to digital conversion process. (Tutor: You could ask students where they think each item –sound sample, time, binary values, and amplitude –will go prior to the animation.) - Now that students have an introduction, it’s time to discuss the ‘Sampling’ part of the process. Sampling is associated with time. There are two more terms to learn. (Tutor: Slide 9 is a diagram with animated labels. You could ask students where they think each item - sample interval, sample rate - will go prior to the animation.) Ask students to complete rows 12 to 13 of Activity 1. (Tutor: The normal sample rate for audio is 44.1kHz i.e. 44,100 samples per second. We are only going to use a very big scale for illustration purposes.) - Now, it is time for the second step, ‘Quantisation’ (Slide 10). It is about the mapping of the amplitude to the binary value on the Y-axis. - Slide 11 is animated and gives students an opportunity to take readings for samples. (Tutor: You could ask students what they think each reading will be and ask them to convert it to binary. Use this opportunity to check if students can remember the range of signed integers that can be represented in one byte.) Ask students to complete Activity 2 to practise reading the graphs. - Now that the process is covered, we need to think about what the overall digital representation might look like (Slide 12). Discuss with students the fact that the analogue signal is no more, and now is represented with a sequence of steps. - As computer scientists we always want to improve what we’ve done, so ask students if they can think of ways to make the digital representation more accurately reflect the original analogue signal (Slide 13). (Tutor: Ask students to make suggestions based on their understanding. You might need to give them some hints based on the graph, before running the animation.) - Wrap up (Slides 14 &15). Homework - See homework document. | Lesson slides Lesson activities Used in lesson: https://www.youtube.com/watch?v=AdQvgyJ1M10 (3:10) Used in homework: https://manual.audacityteam.org/man/sample_rates.html https://www.bbc.co.uk/bitesize/guides/z7vc7ty/revision/3 https://www.youtube.com/watch?v=y_ZHE3fyuGE (0:49) https://www.youtube.com/watch?v=TsQL-sXZOLc (5:48) http://musicandcomputersbook.com/chapter2/02_05.php Also of interest: https://www.bbc.co.uk/bitesize/guides/z6qqmsg/revision/9 |